# Institute Of technology,Nirma University

Project Report of

Design and Implementation of a 16-bit ALU with Status Flags using Verilog HDL on FPGA

Submitted to:

Prof.Rutul Patel

Prof. Aakash Mecwan

Submitted By:

Manash Gusani(25BEC505)

Henil Patel(25BEC518)

Subject:FPGA

Semester:4th

Batch:DtoD

Date:13/03/2026

Design and Implementation of a 16-bit ALU with Status Flags using Verilog HDL on FPGA

## 1. Aim

## To design and implement a 16-bit Arithmetic Logic Unit (ALU) using Verilog HDL, simulate its functionality using ModelSim, and implement it on an FPGA board using Quartus.

## 2. Objectives

## To understand the working of an Arithmetic Logic Unit.

## To design combinational logic using Verilog HDL.

## To verify the ALU operations using RTL simulation.

## To implement the ALU on FPGA hardware.

## To observe arithmetic and logical operations using FPGA switches and LEDs.

# 3. Theory

An Arithmetic Logic Unit (ALU) is the fundamental computational component of a processor. It performs arithmetic operations such as addition and subtraction and logical operations such as AND, OR, and XOR.

The ALU takes two input operands and performs the operation selected by a control signal. In this design, a 4-bit selection input (ALU_Sel) determines which operation is performed.

The ALU also generates status flags indicating the condition of the result.

## 4.Status Flags Generated by the ALU

## The ALU produces six status flags that describe the condition of the result after performing arithmetic or logical operations. These flags are important in processors because they help control conditional operations and detect arithmetic conditions such as overflow or zero results.

| ## Flag | ## Name | ## Description |
| --- | --- | --- |
| CF | Carry Flag | Indicates carry out of the most significant bit during addition or borrow during subtraction |
| OF | Overflow Flag | Indicates overflow in signed arithmetic operations |
| PF | Parity Flag | Set when the least significant byte of the result contains an even number of 1s |
| SF | Sign Flag | Indicates the sign of the result (MSB of the result) |
| ZF | Zero Flag | Set when the result of the operation is zero |
| AF | Auxiliary Carry Flag | Indicates carry from bit 3 to bit 4 (used in BCD arithmetic) |

5. Code

module ALU(

input [1:0] A_in,          // user gives 2 MSB bits

input [1:0] B_in,          // user gives 2 MSB bits

input [3:0] ALU_Sel,

input notA,

input notB,

output reg [15:0] Result,

output reg CF, ZF, SF, OF, PF, AF );

Result = temp[15:0];

CF = temp[16];

AF = (A[3] & B[3]) | ((A[3] ^ B[3]) & ~Result[3]);

OF = (A[15] == B[15]) && (Result[15] != A[15]);

end

4'b0001: begin   // SUB

temp = A - B;

Result = temp[15:0];

CF = temp[16];

AF = (A[3] ^ B[3] ^ Result[3]);

OF = (A[15] != B[15]) && (Result[15] != A[15]);

end

4'b0010: Result = A & B;       // AND

4'b0011: Result = A | B;       // OR

4'b0110: Result = A ^ B;       // XOR

4'b0111: Result = ~(A^B);      // XNOR

4'b1000: Result = ~(A & B);    // NAND

4'b1001: Result = ~(A | B);    // NOR

4'b1010: begin                 // Increment

temp = A + 1;

Result = temp[15:0];

CF = temp[16];

end

4'b1011: begin                 // Decrement

temp = A - 1;

Result = temp[15:0];

end

4'b1100: Result = A << 1;      // Shift Left

4'b1001: Result = A >> 1;      // Shift Right

default: Result = 16'b0;

endcase

// Zero Flag

ZF = (Result == 16'b0);

// Sign Flag

SF = Result[15];

// Parity Flag

PF = ~^Result[7:0];

end

end

endmodule

// Construct full 16-bit operands

wire [15:0] A;

wire [15:0] B;

assign A = {A_in, 14'b00000000000000};

assign B = {B_in, 14'b00000000000000};

reg [16:0] temp;

always @(*)

begin

Result = 16'b0;

CF = 0;

OF = 0;

AF = 0;

ZF = 0;

SF = 0;

PF = 0;

if (notA)

begin

Result = ~A;

end

else if (notB)

begin

Result = ~B;

end

else

begin

case(ALU_Sel)

4'b0000: begin   // ADD

temp = A + B;

4'b1101: Result = A >> 1;      // Shift Right

default: Result = 16'b0;

endcase

// Zero Flag

ZF = (Result == 16'b0);

// Sign Flag

SF = Result[15];

// Parity Flag

PF = ~^Result[7:0];

end

end

endmodule

6. ALU Operation Selection Table

| ## Sr. No. | ## Operation | ## ALU_Sel (4-bit) |
| --- | --- | --- |
| 1 | ADD | 0000 |
| 2 | SUB | 0001 |
| 3 | AND | 0010 |
| 4 | OR | 0011 |
| 5 | NOT A | (SW[8] = 1) |
| 6 | NOT B | (SW[9] = 1) |
| 7 | XOR | 0110 |
| 8 | XNOR | 0111 |
| 9 | NAND | 1000 |
| 10 | NOR | 1001 |
| 11 | INC | 1010 |
| 12 | DEC | 1011 |
| 13 | SHIFT LEFT | 1100 |
| 14 | SHIFT RIGHT | 1101 |

7.RTL VIEW

8.TTL VIEW

9. OUTPUT

https://drive.google.com/file/d/1C3430nCYWdqU_0969zL4QbFIdnLu5Fef/view?usp=drive_link

Conclusion

The 16-bit Arithmetic Logic Unit (ALU) was successfully designed using Verilog HDL and its functionality was verified through RTL simulation. Various arithmetic, logical, and shift operations were implemented using the 4-bit control signal ALU_Sel. The simulation results confirmed that all operations produced the correct outputs. The ALU also generated the status flags CF, OF, PF, SF, ZF, and AF, which indicate the condition of the result. The design was then synthesized and implemented on the FPGA board using Quartus. FPGA switches were used to provide input values and select operations, while LEDs displayed the output results. Both simulation and hardware implementation confirmed the correct operation of the ALU. Therefore, the designed ALU can be used as an important building block in digital systems and processor architectures.